Level 3 Progress Report

Resilient RTK for Disaster Rescue

UAV Emergency Rescue BDS — RTK Positioning Module  |  Tevin Wills  |  2026-06-04

1.  Summary

Level 3 tests whether the RTK positioning system can support an autonomous rescue mission in a disaster zone where GNSS is progressively and simultaneously degraded. We designed a single compound disaster scenario — not four isolated failures — flew it on a real PX4/Gazebo drone mission, and analysed the result against engineering thresholds for approach navigation and precision landing.

Headline result: RTK held mean positioning error to 0.968 m under sustained multi-factor degradation — about 5x better than the total-failure case (4.776 m), and about 20x the ideal baseline (0.048 m). The system PASSED the approach threshold (92.4% of samples within 2.0 m, target >= 90%) but FELL SHORT of the precision-landing threshold (31.5% within 0.3 m, target >= 80%).

2.  What was done, and why

| What | Why |
| --- | --- |
| Designed a 5-phase compound degradation profile | Real disaster zones degrade on several axes at once; isolated single-fault tests do not represent that |
| Modelled 3 co-occurring degradations: noise 1.5->2.75 m, quality 0.95->0.60, dropouts up to 37.5 s / 90 s | To stress the system the way a collapsing-structure search would |
| Added a dynamic uncertainty output and a mission viability state signal | A path planner needs an honest, time-varying confidence signal, not a fixed accuracy claim |
| Flew the scenario as a real 5-waypoint QGC mission (390 x 789 m) | To validate against true vehicle flight, not a stationary hover |
| Ran a total-failure control and reused the Level 2 ideal run as baseline | To bracket the disaster result between best and worst case |
| Produced 7 analysis figures and cross-validated against the PX4 ULog | To make the result presentation-ready and provably real |

3.  Method (scenario at a glance)

| Phase | Time (s) | GNSS noise | Quality | Dropouts |
| --- | --- | --- | --- | --- |
| Departure | 0-120 | 1.5 m | 0.95 | none |
| Approach | 120-240 | 1.5 -> 2.5 m | 0.95 -> 0.60 | 5 s / 60 s |
| Search Zone | 240-600 | 2.75 m | 0.60 | 37.5 s / 90 s |
| Landing | 600-660 | 2.5 m | 0.60 | none |
| Exit | 660-780 | 2.5 -> 1.5 m | 0.65 -> 0.90 | none |

Viability states: LANDING_VIABLE <= 0.3 m  ·  APPROACH_VIABLE <= 2.0 m  ·  DEGRADED <= 4.0 m  ·  INSUFFICIENT > 4.0 m.

4.  What was successfully confirmed

RTK degrades gracefully, not catastrophically — mean error 0.968 m (compound) vs 4.776 m (no corrections), ~5x more accurate even under disaster conditions. (Fig 2, Fig 5)

Approach navigation is viable — 92.4% of approach-phase samples within 2.0 m, beating the >= 90% target. (Fig 6)

The uncertainty signal is honest — reported rtk_std rises and falls with actual error (0.03 -> 4.6 m), unlike the flat Level 2 fixed value. (Fig 3)

The viability state machine behaves correctly — downgrades during dropouts, recovers to LANDING_VIABLE as corrections return. (Fig 4)

The data is real and self-consistent — the PX4 ULog (409 MB) and analysis CSV trace the same 5-waypoint trajectory. (Fig 7)

5.  Honest limitations

Precision landing not met: only 31.5% of landing samples within 0.3 m (target 80%) — reflects RTK Float, not Fixed, during degradation. The key open engineering gap.

Run altitude mismatch: the compound run cruised at 50 m throughout while the total-failure run descended to ~20 m, so the runs are not strictly identical flights. Per-run statistics are unaffected.

Phase windows are time-reconstructed and lead the actual degradation onset by a few tens of seconds; Exit slightly overruns the designed 780 s.

Baseline is a different (shorter) Level 2 mission — used as an accuracy-only reference, noted as such.

6.  Next task

Close the landing gap — implement an RTK re-initialisation / Fixed-lock recovery protocol on entering the landing phase, or extend the landing window for re-convergence.

Re-fly both Level 3 runs with one identical mission (ideally including the WP4 20 m descent) so the runs are strictly comparable.

Phase 3 — RTKLIB validation: post-process real BeiDou RINEX data through RTKLIB and compare measured SPP / Float / Fixed accuracy against the simulation parameters.

Push Level 3 work to the team GitHub once the above are scheduled.

7.  Figure index

| Fig | File | Purpose |
| --- | --- | --- |
| 1 | l3_compound_scenario_profile.png | The imposed 5-phase degradation model |
| 2 | l3_error_over_time.png | RTK vs raw error over the mission (log) |
| 3 | l3_uncertainty_over_time.png | Dynamic vs fixed uncertainty (log) |
| 4 | l3_viability_timeline.png | Mission viability state + error (log) |
| 5 | l3_three_run_comparison.png | Baseline vs compound vs total failure |
| 6 | l3_mission_phase_accuracy.png | Accuracy & threshold pass/fail per phase |
| 7 | l3_ulog_crossval.png | ULog cross-validation (trajectory/error/altitude) |